EXP NO:05	DATE:

DESIGN PCB LAYOUT OF SINGLE-STAGE AMPLIFIER USING KICAD

AIM

To create a schematic of a single-stage amplifier, design its PCB layout using industry-standard tools (e.g., KiCAD), and prepare the design for fabrication.

SOFTWARE REQUIRED

- KiCAD

THEORY

A single-stage amplifier consists of a single transistor with associated biasing and coupling components to amplify a small input signal. It provides voltage gain and is commonly used in preamplifier stages.

The amplifier works in common-emitter configuration, which provides good voltage gain. The essential components include:

- Biasing resistors (R1, R2): Set the DC operating point.

- Collector resistor (Rc): Determines the gain.

- Emitter resistor (Re): Provides stability.

- Coupling capacitors (C1, C2): Block DC components from input/output.

- Bypass capacitor (Ce): Enhances gain by reducing emitter degeneration.

The gain of the amplifier is given by:

where β is the transistor current gain.

CIRCUIT DIAGRAM

Waveforms:

PROCEDURE

Step 1: Design and Simulation

- Select a transistor (BC107) and determine biasing values.

- Draw the schematic using simulation software (KICAD).

- Run the simulation to check gain and performance.

Step 2: Breadboard Implementation

- Assemble the circuit on a breadboard.

- Apply a small AC signal (e.g., 1kHz sine wave).

- Measure the output waveform using an oscilloscope.

- Verify the gain and functionality.

Step 3: PCB Design

- Open KiCad/Eagle/Altium and create a new PCB project.

- Import the schematic and assign footprints to components.

- Arrange components to minimize noise and enhance performance.

- Route the PCB tracks, ensuring proper grounding.

- Generate Gerber files for PCB fabrication.

Step 4: PCB Fabrication

- Print the PCB layout on a copper-clad board using a toner transfer method.

- Etch the board using Ferric Chloride (FeCl₃) solution.

- Drill holes and solder components onto the PCB.

PRECAUTIONS

- Ensure correct polarity of capacitors.

- Use heat sinks if required for high-power applications.

- Check PCB connections before soldering.

OBSERVATIONS

- Compare the simulated vs. practical gain.

- Measure input and output signals using an oscilloscope.

- Observe distortion, noise, and stability.

RESULT

Thus the schematic of a single-stage amplifier was designed and its PCB layout was designed using industry-standard tools (e.g., KiCAD) and the design for fabrication was prepared.